|  | Simulation of cylinder Date: Thursday, December 13, 2012 Designer: Solidworks Study name: SimulationXpress Study Analysis type: Static. Table of Contents Description 1 Assumptions 2 Model Information 2 Material Properties 3 Loads and Fixtures 3 Mesh Information 4 Study Results 6 Conclusion 8. |
| --- | --- |
| # Description No Data |  |

# Assumptions

# Model Information Model name: cylinder Current Configuration: Default. Solid Bodies. Document Name and Reference; Treated As; Volumetric Properties; Document Path/Date Modified. Boss-Extrude1; Solid Body; Mass:3.23175 kg Volume:0.00119694 m^3 Density:2700 kg/m^3 Weight:31.6711 N; C:\Users\Arko\Desktop\Tesla\HABEX Project\HABEX 2\ENV Chamber\cylinder.SLDPRT Dec 13 23:08:46 2012.

# Material Properties Model Reference; Properties; Components. Name:; 6061-T6 (SS). Model type:; Linear Elastic Isotropic. Default failure criterion:; Max von Mises Stress. Yield strength:; 2.75e+008 N/m^2. Tensile strength:; 3.1e+008 N/m^2.; SolidBody 1(Boss-Extrude1)(cylinder).

# Loads and Fixtures Fixture name; Fixture Image; Fixture Details. Fixed-1; Entities:; 2 face(s). Type:; Fixed Geometry.. Load name; Load Image; Load Details. Pressure-1; Entities:; 1 face(s). Type:; Normal to selected face. Value:; 14. Units:; psi..

# Mesh Information Mesh type; Solid Mesh. Mesher Used:; Standard mesh. Automatic Transition:; Off. Include Mesh Auto Loops:; Off. Jacobian points; 4 Points. Element Size; 0.488471 in. Tolerance; 0.0244236 in. Mesh Quality; High. ## Mesh Information - Details Total Nodes; 15029. Total Elements; 7419. Maximum Aspect Ratio; 6.3767. % of elements with Aspect Ratio < 3; 97.6. % of elements with Aspect Ratio > 10; 0. % of distorted elements(Jacobian); 0. Time to complete mesh(hh;mm;ss):; 00:00:03. Computer name:; ARKO-PC.

# Study Results Name; Type; Min; Max. Stress; VON: von Mises Stress; 252071 N/m^2 Node: 1333; 1.81689e+006 N/m^2 Node: 14971. cylinder-SimulationXpress Study-Stress-Stress. Name; Type; Min; Max. Displacement; URES: Resultant Displacement; 0 mm Node: 1; 0.00210212 mm Node: 38. cylinder-SimulationXpress Study-Displacement-Displacement. Name; Type. Deformation; Deformed Shape. cylinder-SimulationXpress Study-Displacement-Deformation. Name; Type; Min; Max. Factor of Safety; Max von Mises Stress; 151.358 Node: 14971; 1090.96 Node: 1333. cylinder-SimulationXpress Study-Factor of Safety-Factor of Safety.

# Conclusion